Thesis Topic:

Interfacing a Two-Level Silicon Carbide Inverter to a Rapid Control Prototyping Platform

Topics to Cover:

- Wolfspeed 2-Level Silicon Carbide Inverter

  - Explanation

    - 800V level

      - Background on market technology

    - Why is Wolfspeed developing this

    - Why do we want to connect Ultrazohm

- Ultrazohm

  - Very briefly as everyone is probably quite familiar with it

- Outline Goal: To interface the Wolfspeed inverter with Ultrazohm - instead of the TI controller card

- Steps Taken until now:

  - Block diagram - identifying the signals the control card is getting from the inverter in the first place

    - We don’t include RSLV_PWM, 3-phase voltage conditioning and SIN/COS signals

      - FIGURE OUT WHY!

    - For all of the signals we determines requirements (such as frequency bandwidth, voltage levels, current levels)

  - LTspice simulations of all the important signal pathways

    - To determine voltage & current levels

      - Ex. There is a bias of 1.68V in the current conditioning because of external 3.3V source

    - Fully Differential Op amps used to do differential analog signaling

      - Because current conditioning is the most important for control - WHY??

    - Defining different voltage levels needed on the interface board

  - Altium

    - Schematic design

      - Power - LDO for 3.3V level, Voltage references for 2.5V and 1.7V level - why did we not also use a voltage reference for the 3.3V level?

        - Making sure that the current budget is kept in mind and fulfilled - 5V 1A input (from the inverter - check again to make sure the correct component that is supplying this)

        - Current pull of

          - Op amp

          - Receivers/Transmitters & dual inverter

          - AND IC

          - Etc.

      - Overcurrent detection - x2 IC of 3 pairs of AND gates - so in the end there is only one signal coming out from the 6 inputs

      - OpAmps with bias

        - Adjusting feedback to the 5V TTL level of the connected analog card - CHECK AND MAKE SURE

      - Decoupling capacitors

      - Ferrite Beads

      - LEDs for indication that it is working

    - PCB Layout

      - Layout finished under review

Next Steps:

- Ordering PCB

- Soldering components - WHAT COMPONENTS WILL I HAVE TO SOLDER ON?

  - Receivers/transmitters

  - Etc.

- Signal Test PCB

  - To test the integrity of the signals

    - We will have another PCB with the connectors signals separated in such a way that we can measure it

- Hook up to Wolfspeed Inverter and coupled motor in the Lab to Test if the interface works!