<Things I did>

PCB Stackup Design

Things to keep in mind

Signal Layer: contains predominantly signal carrying traces

Power/GND Layers: Typically completely solid copper pours over entire layer

- For low-speed and low bandwidth systems you can route power with traces on signal layers

Power supplies needed:

Fully Differential Op-Amp

- VCC5V

- VOC2.5V

- Bias1.65V

- GND

Receivers: Need to check what VCC is – discussed with Eyke, it should be

- VCC3.3V

- GND

Transmitters:

- VCC3.3V

- GND

Overcurrent Detection:

- VCC3.3V

- GND

  - 2.5V for common mode

  - 1.65V for voltage offset

  - 5V supply for Differential Op-Amp -> Analog Signals

  - 3.3V for Digital signals

Questions:

- On the symbol for U5, there are green x marks indicating that a design rule has been ✅ broken. I didn’t do anything, the footprint was already like that – how do I proceed

- Connector and board shape✅

- Origin – parts cannot be moved beyond in the minus direction✅

- It took hours to get everything imported into PCB from the schematic✅

- I did not understand what you meant by this message:

- For some reason there are now 2 AnalogMeasuremens in BitBucket

- +5V Power Disabled?

- Here the Ports are going out in both directions. Is this the convention?

- What type of IC package should I use? ✅ - SMD (Surface Mount Device)

Through hole mounting or surface mounting?

- For the power schematic

  - Can I use simple voltage dividers or should I use switching voltage regulators?

    - We need 5 power levels: GND, VCC5V, VCC3.3V, VCM2.5V, 1.65V bias

      - Understand how much current we are getting from 5V

        - From inverter board

    - Connection from the connector. The connector_inverter has the 5V inputs but how can I place that in my power schematic? Just with a normal circle style power port?

  - Filter for in the +5V_Power input coming in from the connector_inverter?

    - Or can we assume it is stable enough

Meeting Minutes:

Library settings for all components schematics and footprints

- Design -> Make Schematic Library

  - Search first what this does and then implement

  - Hopefully it will take all the parts from the project and make a library

    - Then link the library directly like I did the uz_users.lib

Change the project name itself as well as the name of the pcb file

Power Supply

- We have 5V from the inverter (we don’t need to have a filter or anything?) There are many methods to get from this 5V to the different voltage levels that we want

  - Linear Regulator

  - Buck converter

- First we need to know what our budget is: we have 5V input from inverter at 1A

- Determine the current needed for each of the components that need different level power supply

  - The actual current pulled during operations and also quiescent current

  - Can look at datasheet and also LTSpice Simulation

  - The total current pulled should not exceed the budget (here ex. 1A)

- It is important that we have many test points

  - Put an external 5V input pin header so that we can cut off the input (ferride beads) from the inverter

    - Ferride beads are really small pass through inductors that can be connected in series to the 5V input from the inverter so that that 5V supply can be cut off

- 5V -> 3.3V: Find linear regulator

  - Check all the 3.3V supplied components and estimate what the currents will be

- 5V -> 2.5V: Find linear regulator

  - Check simulation for current values

- Choose a linear regulator (look up how and what points should be kept in mind when doing this)

AND Gates, use SMD surface mount✅

https://www.ti.com/product/de-de/SN74HCS08

SN74HCS08

LTSpice troubleshooting -DO NEXT – NEEDED TO ESTIMATE ALL THE CURRENT ASSUMPTIONS