Agenda:

- Bias Voltage Level

  - I calculated it to be around 1.68V but using the LTspice simulation the differential waves looked the best at 1.7V

- Power Schematic

  - Excel with all the Current pull from the different components

    - 5V, 2.5V, 1.65V -> LTspice simulations

      - I assumed that the quiescent current is included in these values

    - For Optical Receiver/Transmitter/Dual Inverter circuits took the max currents specified on the data sheets

    - Transmitter: No direct voltage supply

      - max. 80mA (continuous forward current I_Fdc)

    - Receiver: High Level Supply Current: max 6.3mA at 5.25Vcc

    - Dual Inverter

      - I_cc max 10 uA

  - Using the TSP731 LDO for 2.5V and 1.7V

    - Feedback requires weird resistor combinations

      - Not in ultrazohm library

  - 3.3V LDO – max 300mA output needed?

    - https://www.ti.com/product/TPS7A20 or this??

  - Header pins for external 5V input & Ferrite bead

    - I am confused as to how the ferrite bead will block the inverter power input

      - Have 2x2 pin header

      - 3x ferrite beads so if we don’t want one power level we can unsolder

        - LED indicator for 3.3V rail so we know when it is turned on

- Importing pcb board shape as dwg/dfx

  - Not working because the units don’t match up

- Just realized that different receiver models are being used is this okay?

  - HFBR-2528Z – receiver when on the website it states Broadcom HFBR-2521Z

  - Can look at the data sheet and get the difference

Meeting Minutes:

- Bias voltage 1.7V

  - We can zero the offset and

  - Voltage reference REF35170

    - Look at current output we need

      - Can change feedback resistors on the diff op amp to vary current

- Bias votlage 2.5 VCOM – also use the same voltage reference REF35170

- Dual inverter

  - Push pull output we do not need the 2k pullup resistor -> legacy keep

- Transmitter

  - Do the calculations, and check by simulation – use a red LED for the transmitter

- Gain of differential op amp should be from 3.3 -> 5V (ADC card in Ultrazohm)

  - Resistors of 10k and 6k68 -> investigate

  - This will change the current pull the whole op amp circuitry!!!

Dual inverter

General Decoupling capacitors!

5V and GND 100nF decoupling capacitors

Also for the 1.7V bias 100nF

Also for overcurrent dectection – look at datasheet

Things to do:

- Read about how bode plots are used in electrical engineering

  - And signal processing

Think about by myself:

- Differential Op Amp Feedback – especially when one end is connected to GND